NEC DataBase Tool Explain

- Click APP2.EXE, you will find:

2. Connect PC and DVB. Click, you will find:

Press Start, then open power of DVB, you will find the loader version .then EXIT.

3. Click, you will find:

Select the connecting COM port, then select the main programme “koda1000_new2_all.img” and press the Start, the procession will go on.

4. Get the data file from DVB. Put the DVB on the upgrade condition (MainMenu/installtion/transfer/transfer program+data), click, you will find:

Press Start, the computer will get the date from the receiver and save the file in a target place after the procession is finished:

5. Edit the date file. Click, open the date file which you save on step 4, then edit the data file, and save the file as other BIN file on a target place.

6. Load the date file, click, and select the BIIN file which you save on step 5:

Press Start, and power off/ on the receiver, you will find:

After finish the procession, restart the receiver, and renew the factory setting operation, the new data can be used now.

7. The relevant information, click or others buttons, you will find the tool information but you cann’t modify them.